WRF Input Processing (Historical)

- Download ERA5 forcing data from globus (I create a folder on CORI and then go to globus connect to move data to the folder). At present I can download 2 years in scratch folder. The folder contains both netcdf and grib files. So, I have to manually delete the netcdf files as they are not need for creating WRF inputs. I retain the grib files. I create two folder jan-jun and jul-dec for each years as we are creating inputs for six months at a time.

- Once the data is download, I create a folder for a particular year (copy a sample folder that I have created) in the WPS directory (/global/cscratch1/sd/rastogi/WRF/WPS) e.g., I have two sample folders: yearjan-jun, yearjul-dec. Say I am processing 1981, I use the following commands to copy the folders.

cp -r yearjan-jun 1981jan-jun

cp -r yearjul-dec 1981jul-dec

- After creating the folders, I run WPS steps inside each of the two folders

  - Edit the namelist file to change the year and make sure the following line is set to FILE

prefix = 'FILE',

  - Link the ERA5 forcing data

./link_grib.csh  path/ERA5-forcing/1981/jan-jun/*grb

  - Link the correct Vtable

ln -s ./ungrib/Variable_Tables/Vtable.ERA-interim.pl Vtable

  - Do the above steps for 1981jul-dec

  - Then submit the job for ungrib.exe for both the folders using single script for parallel processing. Submit the job using the following script by changing the year

/global/cscratch1/sd/rastogi/WRF/WPS/ungrib_cori.sl_year

  - After the job is finished, Make sure all the 3 hourly FILE:XX files are produced for all the time step listed in the namelist.

  - Go inside each folder then link the SST Vtable

rm Vtable

ln -s ./ungrib/Variable_Tables/Vtable.SST Vtable

  - Edit the following line in namelist to

prefix = 'SST',

  - Submit the ungrib job again

/global/cscratch1/sd/rastogi/WRF/WPS/ungrib_cori.sl_year

  - After ungrib job is complete, make sure all the SST files are produced.

  - Submit job for metgrid.exe using the following script and changing the year

/global/cscratch1/sd/rastogi/WRF/WPS/metgrid_cori.sl_year

  - After the job is finished make sure all the 3hourly met_em*nc files are created.

- After WPS steps are complete, I create inputs for WRF

  - I have two sample folders (yearjan-jun, yearjul-dec) under the following path which I copy for that particular year

/global/cscratch1/sd/rastogi/WRF/WRFv421/test/em_real

  - After that, edit namelist to change the year.

  - Link met_em*nc file from the WPS folder

  - Submit script to run WRF (run_real.sl)

- I have two automated scripts to backup output from WPS and WRF to hpss.

Pouya:

WRF configuration:

We use WRF version 4.2.1. At the beginning of this task we spent a lot of time coming up with an optimal configuration for WRF (12 different configurations were tested). The goal was to select physics schemes that result in the best performance over the CONUS and reach the fasted speed for the simulations. This included trying different physics combinations, time steps, adaptive time step configurations, etc. These efforts resulted in the bellow physics and a final namelist we are using for WRF (attached).

The final parametrizations of physical processes in WRF include:

- physics_suite = 'CONUS'

- Microphysics: Thompson (mp_physics = 8)

- cumulus parameterization: Tiedtke (cu_physics = 6)

- longwave radiation: RRTMG (ra_lw_physics = 24)

- Shortwave Radiation: RRTMG (ra_sw_physics = 24)

- Planetary Boundary layer: Mellor-Yamada-Janjic (bl_pbl_physics = 2)

- Surface Layer: Eta similarity (sf_sfclay_physics = 2)

- Land Surface: Noah Land Surface Model (sf_surface_physics = 2)

WRF outputs:

Next, we needed to decide on the output variables required by the IM3 team. We conducted surveys and held discussion with different groups and chose the variables to be archived at hourly intervals (you can see the list at https://immm-sfa.atlassian.net/wiki/spaces/IP/pages/1979809807/Accessing+Historical+and+Future+IM3+Climate+Forcing)

We also needed to decide on a separate set of variables that will be required to run WRF at a higher resolution over regions of interest. A few tests were done, where CONUS scale WRF outputs where downscaled over LA, to decide on the second set of variables (a much longer list) to be saved at a 3-hourly interval.

The output variables are defined for WRF using myoutfields.txt (attached) that is located in the same folder as wrf.exe.

GHG concentration in WRF:

For each WRF simulation we need to provide GHG concentrations based on the year of simulation and the (SSP) scenario. We have downloaded historical AR6 data from https://esgf-node.llnl.gov/search/input4mips/. Concentrations for 5 GHGs (co2 (ppmv), n2o (ppbv), ch4 (ppbv), cfc11 (ppbv), cfc12 (pppbv)) are downloaded and a MATLAB code (GHG_for_WRF_SSP585_v121.m; attached) is used to convert the data to a formant recognizable to WRF. In the WRF folder CAMtr_volume_mixing_ratio is linked to the file created (this is done in the s_restart_V6 code).

Operational steps:

Initial set up:

- mkdir CONUS_PGW_WRF_Historical_v1

- cd CONUS_PGW_WRF_Historical_v1

- cp -rL /project/projectdirs/m1517/Pouya/PROJECT_2021_IM3_Phase_2/WRF/WRFv421_v4ghg_fast/test/em_real/* .

- ln -sf /global/cscratch1/sd/rastogi/WRF/wrf_inputfiles_histruns/1979/jan-mar/wrfbdy_d01 .

- ln -sf /global/cscratch1/sd/rastogi/WRF/wrf_inputfiles_histruns/1979/jan-mar/wrffdda_d01 .

- ln -sf /global/cscratch1/sd/rastogi/WRF/wrf_inputfiles_histruns/1979/jan-mar/wrflowinp_d01 .

- ln -sf /global/cscratch1/sd/rastogi/WRF/wrf_inputfiles_histruns/1979/input-nolakes/wrfinput_d01 .

Restarting the code:

We run WRF for one week at a time (except for one instant of 8 days in normal years and two instants of 8 days for leap years to make sure the code restart at the beginning of month when new input files are linked).

We have a code (s_restart_V6; attached) that links to new input files and updates namelist to reflect the new start time. It also checks if the previous run was successful by comparing the start date on the namelist and the last restart file created. If the code was not successful the code turns off the adaptive time step and uses a smaller time step to get passed the crash point.

Submitting jobs to Cori:

We use a code (wrf_Haswell_7nodes_pureMPI_V6_reservation.sl; attached) that is submitted for the maximum time we are allowed (varies depending on the maintenance schedule; ~ 10 days) on the reserved nodes. This code runs the s_restart_V6 first and then runs wrf.exe for every week of the simulations. This code repeats these steps until the time limit on Cori is reached.

Archiving data:

Hourly data (e.g., wrfout_d01_1990-04-16_01:00:00) is manually copied to /global/cfs/cdirs/m2702/gsharing/CONUS_TGW_WRF_Historical which is linked to a Globus endpoint at: https://app.globus.org/file-manager?origin_id=c296b088-b769-11eb-afd8-e1e7a67e00c1&origin_path=%2F

Both hourly and 3-hourly data (e.g., wrfout_d01_1982-10-29_00:00:00_3hourly) are compressed (two tar files for each year) which are then stored on HPSS manually.